# wBaseline

RHS_Upr_OB_pickup = [538.53,351]; #RHS Upper OB pickup point

RHS_Lwr_OB_pickup = [564.85,171.5]; #RHS Lower OB pickup point

RHS_Upr_IB_pickup = [304.29,348.38]; #RHS Upper IB pickup point

RHS_Lwr_IB_pickup = [227.28,149.35]; #RHS Lower IB pickup point

static_camber = -1.5; #In degrees

UWB_length = 234.254625; #Upper wishbone length in front view

LWB_length = 418.9412262; #Lower wishbone length in front view

UWB_angle = 0.6408328324; #Upper wishbone angle to horizontal. Anti-clockwise positive

LWB_angle = -7.910191806; #Lower wishbone angle to horizontal. Anti-clockwise positive

Baseline kingpin angle is: -8.34˚.

Baseline scrub radius is: 10.0 mm.

Baseline roll centre height is: 164.46 mm.

# RDW_Geom001

RHS_Upr_OB_pickup = [538.53,315]; #RHS Upper OB pickup point

RHS_Lwr_OB_pickup = [564.85,130]; #RHS Lower OB pickup point

RHS_Upr_IB_pickup = [299.98,200]; #RHS Upper IB pickup point

RHS_Lwr_IB_pickup = [215,70]; #RHS Lower IB pickup point

static_camber = -1.5; #In degrees

UWB_length = 264.8; #Upper wishbone length in front view

LWB_length = 354.94; #Lower wishbone length in front view

UWB_angle = 25.739629449782196; #Upper wishbone angle to horizontal. Anti-clockwise positive

LWB_angle = 9.732215290781069; #Lower wishbone angle to horizontal. Anti-clockwise positive

The motivation behind this geometry was to lower the ride height to around 50-70 mm and return the baseline roll centre to a sensible position above the ground plane.

|  | Baseline | New |
| --- | --- | --- |
| Suspension Data | Baseline kingpin angle is: -8.34˚. Baseline scrub radius is: 10.0 mm. Baseline roll centre height is: 164.46 mm. | Baseline kingpin angle is: -8.1˚. Baseline scrub radius is: 16.65 mm. Baseline roll centre height is: 18.62 mm. |
| Geometry Changes in Roll |  |  |
| Roll Centre Migration |  |  |
| RHS Camber Angle vs Roll |  |  |
| LHS Camber Angle vs Roll |  |  |
| Geometry Changes in Bump |  |  |
| Scrub Radius vs Bump |  |  |
| Camber Angle vs Bump |  |  |

## Conclusions:

- Roll centre position and migration is sensible. Lateral migration could be improved but would be acceptable.

- RHS camber changes in bump show a strange characteristic and this needs further investigation with other geometries.

- LHS camber recovery in roll is improved – but potentially “too much” as it loses out in bump.

- Camber changes in bump are much more pronounced but this is an expected result of the more aggressive geometry. It is beyond an acceptable limit and needs to be reduced.

- Scrub radius in bump is less linear but reaches a smaller peak value.

## Next steps:

- Try lengthening the upper wishbone to see the effect on camber.

- Investigate moving the OB points. Can we have a similar scrub radius and reduce the inclination of the wishbones to something more conventional?

# JDRS_Geom001

The baseline geometry is FDW_Geom001. The change in this geometry is a less aggressive angle downwards.

RHS_Upr_OB_pickup = [528,355]; #RHS Upper OB pickup point

RHS_Lwr_OB_pickup = [565,140]; #RHS Lower OB pickup point

RHS_Upr_IB_pickup = [305,290]; #RHS Upper IB pickup point

RHS_Lwr_IB_pickup = [215,110]; #RHS Lower IB pickup point

static_camber = -1.5; #In degrees

UWB_length = 234.254625; #Upper wishbone length in front view

LWB_length = 418.9412262; #Lower wishbone length in front view

UWB_angle = 0.6408328324; #Upper wishbone angle to horizontal. Anti-clockwise positive

LWB_angle = -7.910191806; #Lower wishbone angle to horizontal. Anti-clockwise positive

|  | Baseline | New |
| --- | --- | --- |
| Suspension Data | Baseline kingpin angle is: -8.1˚. Baseline scrub radius is: 16.65 mm. Baseline roll centre height is: 18.62 mm. | Baseline kingpin angle is: -9.59 ˚. Baseline scrub radius is: 11.5mm Baseline roll centre height is: 23.81mm |
| Geometry Changes in Roll |  |  |
| Roll Centre Migration |  |  |
| RHS Camber Angle vs Roll |  |  |
| LHS Camber Angle vs Roll |  |  |
| Geometry Changes in Bump |  |  |
| Scrub Radius vs Bump |  |  |
| Camber Angle vs Bump |  |  |

## Conclusion

- Camber characteristic is a bit more reasonable, with good camber recovery still

- Lower OB BJ is higher to prevent clashes

- Better characteristic in bump

- Roll centre is stable

- The downside is that ride height is not as low as we would want

## Next Steps

- Try to lower the ride height further with a more aggressive setup

# JDRS_Geom002

The baseline geometry is FDW_Geom001. The change in this geometry is a less aggressive angle downwards, but slightly more aggressive than JDRS_Geom001

RHS_Upr_OB_pickup = [528,355]; #RHS Upper OB pickup point

RHS_Lwr_OB_pickup = [565,140]; #RHS Lower OB pickup point

RHS_Upr_IB_pickup = [305,260]; #RHS Upper IB pickup point

RHS_Lwr_IB_pickup = [215,90]; #RHS Lower IB pickup point

static_camber = -1.5; #In degrees

UWB_length = X; #Upper wishbone length in front view

LWB_length = X; #Lower wishbone length in front view

UWB_angle = X; #Upper wishbone angle to horizontal. Anti-clockwise positive

LWB_angle = -X; #Lower wishbone angle to horizontal. Anti-clockwise positive

|  | Baseline | New |
| --- | --- | --- |
| Suspension Data | Baseline kingpin angle is: -8.1˚. Baseline scrub radius is: 16.65 mm. Baseline roll centre height is: 18.62 mm. | Baseline kingpin angle is: -9.76 ˚. Baseline scrub radius is: 10.91mm Baseline roll centre height is: 16.64mm |
| Geometry Changes in Roll |  |  |
| Roll Centre Migration |  |  |
| RHS Camber Angle vs Roll |  |  |
| LHS Camber Angle vs Roll |  |  |
| Geometry Changes in Bump |  |  |
| Scrub Radius vs Bump |  |  |
| Camber Angle vs Bump |  |  |

## Conclusions

- Lower ride height

- Camber recovery is great, with a normal characteristic

- Roll centre stable

- Bump characteristic is marginally better

# JDRS_Geom003

The baseline geometry is JDRS_Geom001. The change in this geometry is a slight extension of the upper wishbone

RHS_Upr_OB_pickup = [528.53,355]; #RHS Upper OB pickup point

RHS_Lwr_OB_pickup = [564.85,140]; #RHS Lower OB pickup point

RHS_Upr_IB_pickup = [300,285]; #RHS Upper IB pickup point

RHS_Lwr_IB_pickup = [215,110]; #RHS Lower IB pickup point

static_camber = -1.5; #In degrees

UWB_length = X; #Upper wishbone length in front view

LWB_length = 418.9412262; #Lower wishbone length in front view

UWB_angle = X; #Upper wishbone angle to horizontal. Anti-clockwise positive

LWB_angle = -7.910191806; #Lower wishbone angle to horizontal. Anti-clockwise positive

|  | Baseline | New |
| --- | --- | --- |
| Suspension Data | Baseline kingpin angle is: -9.59 ˚. Baseline scrub radius is: 11.5mm Baseline roll centre height is: 23.81mm | Baseline kingpin angle is: -9.59˚. Baseline scrub radius is: 11.5 mm. Baseline roll centre height is: 29.47 mm. |
| Geometry Changes in Roll |  |  |
| Roll Centre Migration |  |  |
| RHS Camber Angle vs Roll |  |  |
| LHS Camber Angle vs Roll |  |  |
| Geometry Changes in Bump |  |  |
| Scrub Radius vs Bump |  |  |
| Camber Angle vs Bump |  |  |

## Conclusion

- Camber recovery is even better

- Better characteristic in bump (around about 0 camber at maximum rebound)